Recap:

Overall

- Our group had been working on two stethoscope devices, the Littmann 3200 and our own stethoscope prototype called “splits”

Littmann 3200

- Java scripts and functions were developed to control certain features of the Littmann 3200

  - Development was done under a development license provided by 3M

  - Scripts and functions were developed using the Eclipse Java IDE

Splits

- The “splits” prototype consisted of commercially available products, including microphones, audio-jack splitters, Bluetooth modules, and ear-buds

  - The “splits” prototype was capable of recording and playing audio over the Bluetooth module (not simultaneously)

  - MATLAB scripts, functions, and a graphical user interface (GUI) were developed to operate the prototype

  - Using 3D printers, fixtures were designed to hold the components of the “splits” prototype together

Audio Analysis

- MATLAB was also implemented to develop spectral analysis algorithms

  - These algorithms were designed to detect acoustic features of heart sounds

Update:

Overall

- Our group focused on the development of more advanced prototypes by 3D printing stethoscope parts such as the diaphragm

- Additionally, our team began acquiring and testing components that can replicate and surpass the features of the Littmann 3200

  - Modern microcontrollers can be used to record, analyze, and playback sound waves over Bluetooth antennas

Splits

- Successfully developed a functional 3D printed stethoscope diaphragm (Figure 1)

  - The additive manufacturing process allows for the modification of the design in order to save material, maintain functionality, and thus reduce cost while increasing performance (Figure 1)

    - Designs can be made using a sparse infill in order to save material and space (Figure 1)

    - Wall thicknesses can be increased to avoid acoustic loss and increase the performance of the stethoscope (Figure 1)

- Researched sound acquisition, analysis, and playback on different commercially-available, portable microcontrollers and components (Table 1)

  - Microcontrollers like the Arduino Pro Mini and the Teensy 3.2 can sample audio at 44kHz, making them capable of even performing voice recognition (Table 1)

  - Both microcontrollers are also Arduino compatible, making all software developed thus far reusable

  - The Electret microphone comes with an embedded amplifier (Table 1)

  - Adafruit (electronics manufacturer and distributer) has published several guides that will be used as reference for our prototype

    - These guides include a portable Arduino “Voice Changer” and a “Fast Fourier Transform” analyzer using the Teensy 3.2

| Component ID | Component Name | Manufacturer or Distributor | Component Type | Measurement or Functionality |
| --- | --- | --- | --- | --- |
| DEV-11114 | Arduino Pro Mini 328 - 3.3V/8MHz | SparkFun | Microcontroller | Control circuits |
| ADA-2756 | Teensy 3.2 3.3V | Adafruit | Microcontroller | Control circuits |
| ADA-1063 | Electret Microphone with Amplifier | Adafruit | Microphone | Record Sound |

Moving Forth:

- Our team will replicate the current relevant guides for sound manipulation and analysis

- Our team will develop a protocol for Bluetooth transfer of sound signals

- Our team will develop an stethoscope prototype capable of recording, analyzing, and playing back sound